BÀI BÁO CÁO TUẦN 10

Họ và tên: Nguyễn Trọng Khánh Duy

MSSV: 20210284

Assignment 1:

CODE:

.eqv SEVENSEG_LEFT 0xFFFF0011 # Dia chi cua den led 7 doan trai.

.eqv SEVENSEG_RIGHT 0xFFFF0010 	# Dia chi cua den led 7 doan phai

.text

main:

# MSSV 20210284 => 2 so cuoi la 84

# so 8 khi chuyen sang den led 7 doan : 0111 1111 (bit) = 7F (hex)

# so 4 khi chuyen sang den led 7 doan : 0110 0110 (bit) = 66 (hex)

li 	$a0, 0x7F 	# set value for segments

jal 	SHOW_7SEG_LEFT 	# show

nop

li 	$a0, 0x66 	# set value for segments

jal 	SHOW_7SEG_RIGHT # show

nop

exit: 	li 	$v0, 10

syscall

endmain:

SHOW_7SEG_LEFT:

li 	$t0, SEVENSEG_LEFT # assign port's address

sb 	$a0, 0($t0) 	# assign new value

nop

jr 	$ra

nop

SHOW_7SEG_RIGHT:

li 	$t0, SEVENSEG_RIGHT # assign port's address

sb 	$a0, 0($t0) 	# assign new value

nop

jr 	$ra

nop

Kết quả:

Assignment 2

CODE:

.eqv SEVENSEG_LEFT 0xFFFF0011 # Dia chi cua den led 7 doan trai.

.eqv SEVENSEG_RIGHT 0xFFFF0010 	# Dia chi cua den led 7 doan phai

.data

nhapso:	.asciiz "Nhap so nguyen n = "

.text

main:

li	$v0,4

la	$a0,nhapso

syscall

li	$v0, 5

syscall

add	$s1, $v0, $zero	# n = $s1

div	$s2, $s1, 10

mfhi	$t1		# so hang don vi

div	$s3, $s2, 10

mfhi	$t2		# so hang chuc

addi	$s4, $zero, -1	# khoi tao check = -1

add	$t3, $zero, $t2 # $t3 = so hang chuc

Check_so:

So_0:	bne	$t3, 0, So_1

li	$a0, 0x3F

j	in_so

So_1:	bne	$t3, 1, So_2

li	$a0, 0x06

j	in_so

So_2:	bne	$t3, 2, So_3

li	$a0, 0x5B

j	in_so

So_3:	bne	$t3, 3, So_4

li	$a0, 0x4F

j	in_so

So_4:	bne	$t3, 4, So_5

li	$a0, 0x66

j	in_so

So_5:	bne	$t3, 5, So_6

li	$a0, 0x6D

j	in_so

So_6:	bne	$t3, 6, So_7

li	$a0, 0x7D

j	in_so

So_7:	bne	$t3, 7, So_8

li	$a0, 0x07

j	in_so

So_8:	bne	$t3, 8, So_9

li	$a0, 0x7F

j	in_so

So_9:	bne	$t3, 9, exit

li	$a0, 0x6F

in_so:

beq	$s4, $zero, in_so_don_vi# if check = -1 in so hang chuc

# if check = 0 in so hang don vi

in_so_chuc:

jal 	SHOW_7SEG_LEFT 	# show

nop

addi	$s4, $s4, 1

add	$t3, $zero, $t1 # $t3 = so hang don vi

j Check_so

in_so_don_vi:

jal 	SHOW_7SEG_RIGHT # show

nop

exit: 	li 	$v0, 10

syscall

endmain:

SHOW_7SEG_LEFT:

li 	$t0, SEVENSEG_LEFT # assign port's address

sb 	$a0, 0($t0) 	# assign new value

nop

jr 	$ra

nop

SHOW_7SEG_RIGHT:

li 	$t0, SEVENSEG_RIGHT # assign port's address

sb 	$a0, 0($t0) 	# assign new value

nop

jr 	$ra

nop

Kết quả:

TH1: Khi nhập số có 1 chữ số:

TH2: Khi nhập một số có 2 chữ số:

TH3: Khi nhập một số có 3 chữ số:

Assignment 3

CODE:

.eqv SEVENSEG_LEFT 0xFFFF0011 # Dia chi cua den led 7 doan trai.

.eqv SEVENSEG_RIGHT 0xFFFF0010 	# Dia chi cua den led 7 doan phai

.data

nhapkytu:	.asciiz "Nhap ky tu: "

.text

main:

li	$v0,4

la	$a0,nhapkytu

syscall

li	$v0, 12

syscall

add	$s1, $v0, $zero	# n = $s1

div	$s2, $s1, 10

mfhi	$t1		# so hang don vi

div	$s3, $s2, 10

mfhi	$t2		# so hang chuc

addi	$s4, $zero, -1	# khoi tao check = -1

add	$t3, $zero, $t2 # $t3 = so hang chuc

Check_so:

So_0:	bne	$t3, 0, So_1

li	$a0, 0x3F

j	in_so

So_1:	bne	$t3, 1, So_2

li	$a0, 0x06

j	in_so

So_2:	bne	$t3, 2, So_3

li	$a0, 0x5B

j	in_so

So_3:	bne	$t3, 3, So_4

li	$a0, 0x4F

j	in_so

So_4:	bne	$t3, 4, So_5

li	$a0, 0x66

j	in_so

So_5:	bne	$t3, 5, So_6

li	$a0, 0x6D

j	in_so

So_6:	bne	$t3, 6, So_7

li	$a0, 0x7D

j	in_so

So_7:	bne	$t3, 7, So_8

li	$a0, 0x07

j	in_so

So_8:	bne	$t3, 8, So_9

li	$a0, 0x7F

j	in_so

So_9:	bne	$t3, 9, exit

li	$a0, 0x6F

in_so:

beq	$s4, $zero, in_so_don_vi# if check = -1 in so hang chuc

# if check = 0 in so hang don vi

in_so_chuc:

jal 	SHOW_7SEG_LEFT 	# show

nop

addi	$s4, $s4, 1

add	$t3, $zero, $t1 # $t3 = so hang don vi

j Check_so

in_so_don_vi:

jal 	SHOW_7SEG_RIGHT # show

nop

exit: 	li 	$v0, 10

syscall

endmain:

SHOW_7SEG_LEFT:

li 	$t0, SEVENSEG_LEFT # assign port's address

sb 	$a0, 0($t0) 	# assign new value

nop

jr 	$ra

nop

SHOW_7SEG_RIGHT:

li 	$t0, SEVENSEG_RIGHT # assign port's address

sb 	$a0, 0($t0) 	# assign new value

nop

jr 	$ra

nop

Kết quả:

Nhập ký tự thường:

Nhập ký tự in hoa:

Assignment 4

CODE:

.eqv MONITOR_SCREEN 0x10010000 #Dia chi bat dau cua bo nho man hinh

.eqv RED 0x00FF0000 #Cac gia tri mau thuong su dung

.eqv GREEN 0x0000FF00

.eqv BLUE 0x000000FF

.eqv WHITE 0x00FFFFFF

.eqv YELLOW 0x00FFFF00

.text

li 	$k0, MONITOR_SCREEN 	#Nap dia chi bat dau cua man hinh

li	$t2, 2			# khoi tao 2

li	$t4, 8			# khoi tao 8

li	$t1, -1			# i = -1

li	$t4, 0			# j = 0

li 	$s1, 4			# $s1 = 4

add	$k0, $k0, -4		# $k0 = $k0 - 4

FOR:	addi	$t1, $t1, 1		# i = 0, i ++

addi	$t4, $t4, 1		# j++

beq	$t1, 72, EXIT		# i = 72 stop

add	$k0, $k0, $s1		# $k0 += 4

div	$t1, $t2		# i / 2

mfhi	$t3			# $t3 = i % 2

bne	$t4, 8, continue	# j = 8 => i++

li 	$t4, 0			# j = 0

addi	$t1, $t1, 1		# i++

continue:

beq	$t3, $zero, doi_mau

li 	$t0, RED

sw 	$t0, 0($k0)

nop

j	FOR

doi_mau:

li 	$t0, BLUE

sw 	$t0, 0($k0)

nop

j FOR

EXIT:	li 	$v0, 10

syscall

Giải thích code:

- Dùng một biến i chạy từ 0 – 71 thì dừng

- Dùng một biến j chạy từ 1 – 8

- Khi i /2 dư 1 thì sẽ in màu đỏ, i / 2 dư 0 thì in màu xanh

- Vì bảng là 8x8 nên mỗi lần j = 8 sẽ tăng i thêm 1 => i sẽ tăng 2 đơn vị khi xuống dòng => khi xuống dòng i sẽ giữ nguyên màu. Vì có 8 dòng nên sẽ tăng i lên 8 đơn vị nên phải đặt cho i dừng khi i = 72

Kết quả:

Assignment 5

CODE:

.eqv MONITOR_SCREEN 0x10010000

.eqv RED            0x00FF0000

.eqv GREEN          0x0000FF00

.data

x1: .asciiz "Nhap x1: "

y1: .asciiz "Nhap y1: "

x2: .asciiz "Nhap x2: "

y2: .asciiz "Nhap y2: "

error1: .asciiz "Error: x2 phai khac x1. Moi nhap lai!\n"

error2: .asciiz "Error: y2 phai khac y1. Moi nhap lai!\n"

.text

li	$k0, MONITOR_SCREEN

li	$v0, 4

la	$a0, x1

syscall

li	$v0, 5

syscall

move	$s0, $v0

li	$v0, 4

la	$a0, y1

syscall

li	$v0, 5

syscall

move	$s1, $v0

NhapX2: li	$v0, 4

la	$a0, x2

syscall

li	$v0, 5

syscall

move	$s2, $v0

beq	$s2, $s0, Error1

NhapY2: li	$v0, 4

la	$a0, y2

syscall

li	$v0, 5

syscall

move	$s3, $v0

beq	$s3, $s1, Error2

j	Tsugi

Error1: li	$v0, 4

la	$a0, error1

syscall

j 	NhapX2

Error2:	li	$v0, 4

la	$a0, error2

syscall

j 	NhapY2

Tsugi:

slt	$t0, $s0, $s2

slt	$t1, $s1, $s3

beq	$t0, 0, Case3

beq	$t1, 0, Case2

Case1:	add	$v0, $s1, $zero

For1: 	bgt	$v0, $s3, Exit

add	$v1, $s0, $zero

For2: 	bgt	$v1, $s2, EndFor2

beq	$v0, $s1, InVien1

beq	$v0, $s3, InVien1

beq	$v1, $s0, InVien1

beq	$v1, $s2, InVien1

sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, GREEN

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For2

InVien1:	sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, RED

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For2

EndFor2:

add	$v0, $v0, 1

j	For1

Case2:	add	$v0, $s3, $zero

For3: 	bgt	$v0, $s1, Exit

add	$v1, $s0, $zero

For4: 	bgt	$v1, $s2, EndFor4

beq	$v0, $s1, InVien2

beq	$v0, $s3, InVien2

beq	$v1, $s0, InVien2

beq	$v1, $s2, InVien2

sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, GREEN

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For4

InVien2:sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, RED

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For4

EndFor4:

add	$v0, $v0, 1

j	For3

Case3:	beq	$t1, 0, Case4

add	$v0, $s1, $zero

For5: 	bgt	$v0, $s3, Exit

add	$v1, $s2, $zero

For6: 	bgt	$v1, $s0, EndFor6

beq	$v0, $s1, InVien3

beq	$v0, $s3, InVien3

beq	$v1, $s0, InVien3

beq	$v1, $s2, InVien3

sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, GREEN

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For6

InVien3:sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, RED

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For6

EndFor6:

add	$v0, $v0, 1

j	For5

Case4:	add	$v0, $s3, $zero

For7: 	bgt	$v0, $s1, Exit

add	$v1, $s2, $zero

For8: 	bgt	$v1, $s0, EndFor8

beq	$v0, $s1, InVien4

beq	$v0, $s3, InVien4

beq	$v1, $s0, InVien4

beq	$v1, $s2, InVien4

sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, GREEN

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For8

InVien4:sll	$t8, $v0, 6

add	$t8, $t8, $v1

sll	$t8, $t8, 2

li	$a1, RED

add	$a2, $k0, $t8

sw	$a1, 0($a2)

add	$v1, $v1, 1

j	For8

EndFor8:

add	$v0, $v0, 1

j	For7

Exit:	li	$v0, 10

syscall

Kết quả: